| DGS-1100-05 Firmware Version: | 1.10.B009 |
| --- | --- |
| DGS-1100-05 PROM Code Version: | 1.0.5 |
| DGS-1100-08 Firmware Version:: | 1.10.B023 1.10.U001 |
| DGS-1100-08 PROM Code Version:: | 1.0.11 |
| DGS-1100-08P Firmware Version:: | 1.10.B014 1.10.U001 |
| DGS-1100-08P PROM Code Version:: | 1.0.2 |
| DGS-1100-16/24 Firmware Version:: | 1.10.B015 |
| DGS-1100-16/24 PROM Code Version: | 1.0.2 |
| Published: | Apr. 9, 2014 |

These release notes include important information about D-Link switch firmware revisions. Please verify that these release notes are correct for your switch:

- If you are installing a new switch, please check the hardware version on the device label; make sure that your switch meets the system requirement of this firmware version. Please refer to Revision History and System Requirement for detailed firmware and hardware matrix.

- If the switch is powered on, you can check the hardware version by checking the device information page on the web graphic user interface.

- If you plan to upgrade to the new firmware release, please refer to the Upgrade Instructions for the correct firmware upgrade procedure.

For more detailed information regarding our switch products, please refer to Related Documentation.

You can also download the switch firmware and technical documentation from http://tsd.dlink.com.tw.

Content:

Revision History and System Requirement:	2

Upgrade Instructions:	2

Upgrade using Web-UI	2

Upgrade using SmartConsole Utility	3

New Features:	6

Changes:	6

Problem Fixed:	7

Known Issues:	8

Related Documentation:	10

# Revision History and System Requirement:

| Firmware Version | Date | Model | Hardware Version |
| --- | --- | --- | --- |
| Runtime:V1.10.B009 | 9-Apr-2014 | DGS-1100-05 | A1 |
| Runtime:V1.10.B023 | 9-Apr-2014 | DGS-1100-08 | A1 |
| Runtime:V1.10.B014 | 9-Apr-2014 | DGS-1100-08P | A1 |
| Runtime:V1.10.U001 | 9-Apr-2014 | DGS-1100-08 | A1 |
| Runtime:V1.10.U001 | 9-Apr-2014 | DGS-1100-08P | A1 |
| Runtime:V1.10.B022 | 1-Apr-2014 | DGS-1100-08 | A1 |
| Runtime:V1.10.B013 | 1-Apr-2014 | DGS-1100-08P | A1 |
| Runtime:V1.10.B008 | 14-Mar-2014 | DGS-1100-05 | A1 |
| Runtime:V1.10.B021 | 14-Mar-2014 | DGS-1100-08 | A1 |
| Runtime:V1.10.B012 | 14-Mar-2014 | DGS-1100-08P | A1 |
| Runtime:V1.10.B007 | 17-Jan-2014 | DGS-1100-05 | A1 |
| Runtime:V1.10.B011 | 17-Jan-2014 | DGS-1100-08P | A1 |
| Runtime:V1.10.B015 | 17-Jan-2014 | DGS-1100-16 DGS-1100-24 | A1 |
| Runtime:V1.10.B020 | 16-Jan-2014 | DGS-1100-08 | A1 |
| Runtime:V1.10.B010 | 24-Dec-2013 | DGS-1100-08P | A1 |
| Runtime:V1.10.B019 | 24-Dec-2013 | DGS-1100-08 | A1 |
| Runtime:V1.10.B006 | 23-Dec-2013 | DGS-1100-05 | A1 |
| Runtime:V1.10.B009 | 23-Dec-2013 | DGS-1100-08P | A1 |
| Runtime:V1.10.B018 | 20-Dec-2013 | DGS-1100-08 | A1 |
| Runtime:V1.10.B008 | 11-Dec-2013 | DGS-1100-08P | A1 |
| Runtime:V1.10.B017 | 11-Dec-2013 | DGS-1100-08 | A1 |
| Runtime:V1.10.B007 | 28-Nov-2013 | DGS-1100-08P | A1 |
| Runtime:V1.10.B016 | 28-Nov-2013 | DGS-1100-08 | A1 |
| Runtime:V1.10.B014 | 01-Aug-2013 | DGS-1100-16 DGS-1100-24 | A1 |
| Runtime:V1.10.B005 | 08-Aug-2013 | DGS-1100-05 | A1 |
| Runtime:V1.10.B006 | 07-Aug-2013 | DGS-1100-08P | A1 |
| Runtime:V1.10.B013 | 01-Aug-2013 | DGS-1100-16 DGS-1100-24 | A1 |
| Runtime:V1.10.B015 | 29-Jul-2013 | DGS-1100-08 | A1 |
| Runtime:V1.10.B014 | 19-Jul-2013 | DGS-1100-08 | A1 |
| Runtime:V1.10.004 PROM:1.0.5 | 26-Apr-2013 | DGS-1100-05 | A1 |
| Runtime:V1.10.011 PROM:1.0.11 | 18-Mar-2013 | DGS-1100-08 | A1 |
| Runtime:V1.10.011 PROM:1.0.2 |  | DGS-1100-16 DGS-1100-24 | A1 |
| Runtime:V1.10.002 PROM:1.0.4 | 4-Oct-2012 | DGS-1100-05 | A1 |
| Runtime:V1.10.007 PROM:1.0.11 |  | DGS-1100-08 | A1 |
| Runtime:V1.10.008 PROM:1.0.2 |  | DGS-1100-16 DGS-1100-24 | A1 |
| Runtime:V1.10.005 PROM:1.0.2 | 2-Jun-2012 | DGS-1100-16 DGS-1100-24 | A1 |
| Runtime:V1.00.012 PROM:1.0.2 |  | DGS-1100-16 DGS-1100-24 | A1 |
| Runtime:V1.10.002 PROM:1.0.8 |  | DGS-1100-08 | A1 |
| V1.00.009 | 27-Dec-2010 | DGS-1100-16 DGS-1100-24 | A1 |

# Upgrade Instructions:

You can download the firmware from D-Link web site http://tsd.dlink.com.tw, and upgrade through web UI or SmartConsole Utility.

Upgrade using Web-UI

- Connect a workstation installed with java SE runtime environment to any switch port of the device.

- Open the web browser from the workstation and enter the IP address of the switch. The switch’s default IP address is 10.90.90.90.

- Enter administrator’s password when prompted. The password is admin by default.

- To update switch's firmware or configuration file, select Tools > Firmware Backup & Upgrade from the banner.

- Click Browse to browse your inventories for a saved firmware file, and then click Upgrade after selecting the firmware file you want to restore.

- Wait until the “Firmware Upgrade Successful” message pops up and login again after device boots up.

Upgrade using SmartConsole Utility

- Connect a workstation installed with java SE runtime environment to any switch port of the device

2. Execute SmartConsole Utility and click the “Discovery” button to search switches.

3. Single-click on the icon of Monitor column to select the target switch.

4. Click Firmware Upgrade button

5. Click Browse button and select the firmware file (Model name_HW ver._FW ver. .hex) on your local hard drive.

- Enter password and click “Upgrade” button to start firmware upgrade. The default password is “admin”.

7. Once the message popped up, click “OK” button to complete the firmware upgrade

# New Features:

DGS-1100-05:

| Firmware Version | New Features |
| --- | --- |
| V1.10.B009 | NA |
| V1.10.B008 | NA |
| V1.10.B007 | NA |
| V1.10.B006 | NA |
| V1.10.B005 | NA |
| V1.10.004 | NA |
| V1.10.002 | First Release |

DGS-1100-08:

| Firmware Version | New Features |
| --- | --- |
| V1.10.U001 | NA |
| V1.10.B023 | NA |
| V1.10.B022 | Add EEE web setting page. |
| V1.10.B021 | NA |
| V1.10.B020 | NA |
| V1.10.B019 | NA |
| V1.10.B018 | NA |
| V1.10.B017 | NA |
| V1.10.B016 | NA |
| V1.10.B015 | NA |
| V1.10.B014 | NA |
| V1.10.011 | NA |
| V1.10.007 | SmartConsole Utility v3.0 |
| V1.10.002 | First Release |

DGS-1100-08P:

| Firmware Version | New Features |
| --- | --- |
| V1.10.U001 | NA |
| V1.10.B014 | NA |
| V1.10.B013 | Add EEE web setting page. |
| V1.10.B012 | NA |
| V1.10.B011 | NA |
| V1.10.B010 | NA |
| V1.10.B009 | NA |
| V1.10.B008 | NA |
| V1.10.B007 | NA |
| V1.10.B006 | NA |

DGS-1100-16/24:

| Firmware Version | New Features |
| --- | --- |
| V1.10.B015 | NA |
| V1.10.B014 | NA |
| V1.10.B013 | NA |
| V1.10.011 | NA |
| V1.10.008 | SmartConsole Utility v3.0 |
| V1.10.005 | - Management VLAN - Voice VLAN |
| V1.00.009 | First Release |

# Changes:

DGS-1100-05:

| Firmware Version | Changes |
| --- | --- |
| V1.10.B009 | NA |
| V1.10.B008 | NA |
| V1.10.B007 | 1. Add MAC OUI B0C5540~B0C5547 to Surveillance VLAN function. |
| V1.10.B006 | 1. Change DHCP retry Interval to fixed 5 second. 2. Add DHCP retry time setting (Range: 5 - 120, Default: 7 times). |
| V1.10.B005 | NA |
| V1.10.004 (PROM V1.0.5) | - Close UART input for ignore interference from H/W. - If the version of Loader code in the switch is older than "V1.0.5" , It will Upgrade the Loader code after upgraded to this version F/W code and boot up to runtime code. |
| V1.10.002 | First Release |

DGS-1100-08:

| Firmware Version | Changes |
| --- | --- |
| V1.10.U001 | 1. Overwrite old password when restoring configuration. 2. Chande DHCP's default value to Enabled.. |
| V1.10.B023 | 1. Resume DHCP retry default time to 7 times. |
| V1.10.B022 | Modify the EEE's default setting to Disable. |
| V1.10.B021 | NA |
| V1.10.B020 | 1. Add MAC OUI B0C5540~B0C5547 to Surveillance VLAN function. |
| V1.10.B019 | 1. Change DHCP retry default time to 120 times. |
| V1.10.B018 | 1. Change DHCP retry Interval to fixed 5 second. 2. Add DHCP retry time setting (Range: 5 - 120, Default: 7 times). |
| V1.10.B017 | NA |
| V1.10.B016 | NA |
| V1.10.B015 | NA |
| V1.10.B014 | NA |
| V1.10.011 | Forbid to restore the configuration file from certain case. For details, refer to table at “Know Issues” part. |
| V1.10.007 | Move Menu Items "Surveillance VLAN" and "Voice VLAN" to Menu "VLAN". |
| V1.10.002 | First Release |

DGS-1100-08P:

| Firmware Version | Changes |
| --- | --- |
| V1.10.U001 | 1. Overwrite old password when restoring configuration. 2. Chande DHCP's default value to Enabled.. |
| V1.10.B014 | 1. Resume DHCP retry default time to 7 times. |
| V1.10.B013 | Modify the EEE's default setting to Disable. |
| V1.10.B012 | NA |
| V1.10.B011 | 1. Add MAC OUI B0C5540~B0C5547 to Surveillance VLAN function. |
| V1.10.B010 | 1. Change DHCP retry default time to 120 times. |
| V1.10.B009 | 1. Change DHCP retry Interval to fixed 5 second. 2. Add DHCP retry time setting (Range: 5 - 120, Default: 7 times). |
| V1.10.B008 | NA |
| V1.10.B007 | NA |
| V1.10.B006 | NA |

DGS-1100-16/24:

| Firmware Version | Changes |
| --- | --- |
| V1.10.B015 | 1. Add MAC OUI B0C5540~B0C5547 to Surveillance VLAN function. |
| V1.10.B014 | NA |
| V1.10.B013 | NA |
| V1.10.011 | Forbid downgrading FW to previous version. For details, refer to table at “Know Issues” part. |
| V1.10.008 | Move Menu Items "Surveillance VLAN" and "Voice VLAN" to Menu "VLAN". |
| V1.10.005 or later version | Forbid to restore the configuration file from certain case. For details, refer to table at “Know Issues” part. |  |
| V1.10.002 | First Release |

# Problem Fixed:

DGS-1100-05:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.10.B009 | - fix the bug: In some cases, DHCP enabled will lead to the device can't be access and can't be reset by reset button. |
| V1.10.B008 | - D-Track [HQ20140314000007]fix the bug: DHCP Enable, Then Save configuration and power off the device immediately. The device will can't be access and can't be reset by reset button. |
| V1.10.B007 | - D-Track [HQ20140115000007]fix the bug: There are no any dhcp discover packets send to DHCP server, the switch can't be accessed by default IP address or reset button after enable DHCP and upgrade FW from V1.10.B006 |
| V1.10.B006 | NA |
| V1.10.B005 | - D-Track [HQ20130704000014] Fix the bug: can't link up IC+ IP1000A LF 1000Base-T adapter on 1G speed with autonegotiation. |
| V1.10.004 | - Fix the forwarded IGMP snooping packets haven’t tag when IGMP Snooping enabled. - Fix Port base VLAN settings will lost efficacy after reboot. - D-Track [HQ20130110000014] Fix IGMP control packet only be processed when packet is in Management VLAN. - D-Track [HQ20130114000016] Fix the IGMP Group which all members have been leaved is not removed while router port exist. |
| V1.10.002 | First Release |

DGS-1100-08:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.10.U001 | NA |
| V1.10.B023 | - fix the bug: In some cases, DHCP enabled will lead to the device can't be access and can't be reset by reset button. |
| V1.10.B022 | NA |
| V1.10.B021 | - D-Track [HQ20140314000007]fix the bug: DHCP Enable, Then Save configuration and power off the device immediately. The device will can't be access and can't be reset by reset button. |
| V1.10.B020 | - D-Track [HQ20140115000007]fix the bug: There are no any dhcp discover packets send to DHCP server, the switch can't be accessed by default IP address or reset button after enable DHCP and upgrade FW from V1.10.B017 |
| V1.10.B019 | NA |
| V1.10.B018 | NA |
| V1.10.B017 | - The transaction IDs begin generated is same on multiple switches being powered on. |
| V1.10.B016 | - D-Track [HQ20131127000004] Fix the bug: does not attempt to renew its DHCP lease by sending the DHCP request after the lease has expired, rather it begins a whole new DHCP session beginning with the DHCP discover. RFC2131 imply that a DHCP client is required to send a request to renew its lease at 1/2 lease time. |
| V1.10.B015 | - D-Track [HQ20130704000014] Fix the bug: can't link up IC+ IP1000A LF 1000Base-T adapter on 1G speed with autonegotiation. |
| V1.10.B014 | - D-Track [HQ20130715000005] Change the status to "Loop" in “Loopback Detection” setting page, "Error Disabled" in “Port setting” page while port is looped and been disabled. - D-Track [HQ20130715000007] Fixed System up time Error after about 6 hours. |
| V1.10.011 | - Fix the configuration lost when FW upgrade from V1.10.002. - Fix the forwarded IGMP snooping packets haven’t tag when IGMP Snooping enabled. - D-Track [HQ20130114000016] Fix the IGMP Group which all members have been leaved is not removed while router port exist. - Fix port base VLAN settings will lost efficacy after reboot. - D-Track [HQ20130110000014] Fix IGMP control packet only be processed when packet is in Management VLAN. - Fix selfloop port sometime display in block state when port is linkdown. |
| V1.10.007 | - Fix press Refresh button when firmware upgrade by IE will cause packet continuous transmission. - Fix web page: Port Statistics will show incorrect RxBadPkt. - Change DUT's behavior to only reply ARP to Management VLAN when Management VLAN is enabled. - Fix ARP reply FCS error. - D-Track [HQ20130109000003] Fix the serial number is changed after restore the backup file of other switches. |
| V1.10.002 | First Release |

DGS-1100-08P:

| Firmware Version | Problems Fixed |
| --- | --- |
| V1.10.U001 | NA |
| V1.10.B014 | - fix the bug: In some cases, DHCP enabled will lead to the device can't be access and can't be reset by reset button. |
| V1.10.B013 | NA |
| V1.10.B012 | - D-Track [HQ20140314000007]fix the bug: DHCP Enable, Then Save configuration and power off the device immediately. The device will can't be access and can't be reset by reset button. |
| V1.10.B011 | - D-Track [HQ20140115000007]fix the bug: There are no any dhcp discover packets send to DHCP server, the switch can't be accessed by default IP address or reset button after enable DHCP and upgrade FW from V1.10.B017 |
| V1.10.B010 | NA |
| V1.10.B009 | NA |
| V1.10.B008 | - The transaction IDs begin generated is same on multiple switches being powered on. |
| V1.10.B007 | - D-Track [HQ20131127000004] Fix the bug: does not attempt to renew its DHCP lease by sending the DHCP request after the lease has expired, rather it begins a whole new DHCP session beginning with the DHCP discover. RFC2131 imply that a DHCP client is required to send a request to renew its lease at 1/2 lease time. |
| V1.10.B006 | - D-Track [HQ20130704000014] Fix the bug: can't link up IC+ IP1000A LF 1000Base-T adapter on 1G speed with autonegotiation. |

DGS-1100-16/24:

| Firmware Version |  | Problems Fixed |  |
| --- | --- | --- | --- |
| V1.10.B015 | - D-Track [HQ20140115000007]fix the bug: There are no any dhcp discover packets send to DHCP server, the switch can't be accessed by default IP address or reset button after enable DHCP and upgrade FW from V1.10.B017 |  |
| V1.10.B014 | D-Track [HQ20130808000006 ] Fix the bug: The IGMP Snooping function not work in the not management VLAN when Management VLAN Enabled. |  |
| V1.10.B013 | D-Track [HQ20130708000013] Fix the bug: Device doesn't send ARP request to Gateway when packet needs to forward to Gateway. |  |
| V1.10.011 | D-Track [HQ20130205000009] Fix the configuration lost when FW upgrades from V1.00.009 and V1.00.012. |  |
| V1.10.005 |  | NA |  |
| V1.00.012 |  | Fix the configuration lost when FW upgrades from V1.00.009 to V1.10.005. Note: This Firmware will NOT cut in to production. |  |
| V1.00.009 |  | First Release |  |

# Known Issues:

| Firmware Version | Issues | Workaround |
| --- | --- | --- |
| V1.10.004 (For DGS-1100-05) | If power off while upgrading the Loader code, the switch's FW will be damaged. It should be done RMA for repair. | Don’t power off the switch in 30 seconds after upgraded F/W to this version. |
| V1.10.005/V1.10.008 (For DGS-1100-16/24) | If downgrade to V1.00.009/V1.00.012 then upgrade to V1.10.005/V1.10.008, the all ports of switch can’t linkup. | Press on reset button and hold 3 secends at least for fatory resetting the switch. |

DGS-1100-08 configuration and F/W upgrade/downgrade Issue:

| Copy Configuration | Restore To | 1.10.002 | 1.10.007 | 1.10.011 or later version |
| --- | --- | --- | --- | --- |
| Backup From |  |  |  |  |
| V1.10.002 | Result | The SN will be overwritten.* | A part of configurations will be factory reset.** | Forbid restore |
| V1.10.007 | Result | The SN and a part of configurations will be error.*** | Normal | Forbid restore |
| V1.10.011 or later version | Result | Forbid restore | Forbid restore | Normal |

| F/W Upgrade or Downgrade | To | 1.10.002 | 1.10.007 | 1.10.011 or later version |
| --- | --- | --- | --- | --- |
| From |  |  |  |  |
| V1.10.002 | Result | Normal | A part of configurations will be factory reset.** | Normal |
| V1.10.007 | Result | The SN and a part of configurations will be error.*** | Normal | Normal |
| V1.10.011 or later version | Result | The SN, HW version and a part of configurations will be error.**** | Normal | Normal |

Note :

* :The SN only can be recovered throught RMA(Return Material Authorization), and the value of SN not affect any function of the switch. Suggest upgrade F/W to V1.10.011 first then backup configuration.

** :The part of configurations Include System Information, Trap Settings for SmartConsole, Surveillance VLAN and Voice Vlan will be factory reset, should be set again by user. Suggest upgrade F/W to V1.10.011 first then backup configuration.

*** :The SN only can be recovered throught RMA(Return Material Authorization), and the value of SN not affect any function of the switch. The part of configurations Include System Information, Trap Settings for SmartConsole, Surveillance VLAN and Voice Vlan will be factory reset, should be set again by user. Suggest not to downgrade or backup configurations to old F/W.

**** :The SN and HW version will be recovered after upgrade F/W back to V1.10.011 or later version but configurations need to set again.

DGS-1100-16/24 configuration and F/W upgrade/downgrade Issue:

| Copy Configuration | Restore To | 1.00.009 | 1.00.012 | 1.10.005 | 1.10.008 | 1.10.011 or later version |
| --- | --- | --- | --- | --- | --- | --- |
| Backup From |  |  |  |  |  |  |
| V1.00.009 | Result | Normal | Normal | Forbid restore | Forbid restore | Forbid restore |
| V1.00.012 | Result | Normal | Normal | Forbid restore | Forbid restore | Forbid restore |
| V1.10.005 | Result | Forbid restore | Forbid restore | Normal | Normal | Normal |
| V1.10.008 | Result | Forbid restore | Forbid restore | Normal | Normal | Normal |
| V1.10.011 or later version | Result | Forbid restore | Forbid restore | Normal | Normal | Normal |

| F/W Upgrade or Downgrade | To | 1.00.009 | 1.00.012 | 1.10.005 | 1.10.008 | 1.10.011 or later version |
| --- | --- | --- | --- | --- | --- | --- |
| From |  |  |  |  |  |  |
| V1.00.009 | Result | Normal | Normal | The SN and configuration will be factory reset.* | The SN and configuration will be factory reset.* | Normal |
| V1.00.012 | Result | Normal | Normal | Normal | The SN and configuration will be factory reset.* | Normal |
| V1.10.005 | Result | The SN and configuration will be factory reset.** | Normal | Normal | Normal | Normal |
| V1.10.008 | Result | The SN and configuration will be factory reset.** | The SN and configuration will be factory reset.** | Normal | Normal | Normal |
| V1.10.011 or later version | Result | Forbid | Forbid | Forbid | Forbid | Normal |

Note :

*	: When firmware upgrade in this situation, the SN and configuration will be factory reset. Suggest upgrade F/W to V1.10.011 or Later version.

**	: When firmware downgrade in this situation, the SN and configuration will be factory reset. Suggest not to downgrade.

# Related Documentation:

- DGS-1100 Series User Manual

- DGS-1100 Series Getting Started Guide